# PagerDuty Datacenter Demo - Project Summary

# Project Overview

Objective: Design and implement a PagerDuty demo for a datacenter provider integrating BMS (Building Management System) alerts in Niagara format.

Location: Sydney, Australia (Australia/Sydney timezone)

Collaboration Tool: Microsoft Teams (instead of Slack)

# Current Status

✅ Interactive visual diagram created (HTML/React)

✅ Complete Terraform infrastructure code generated

✅ Fixed deprecated alert_grouping_parameters

✅ Fixed intelligent grouping config blocks

⚠️ BLOCKED: Terraform apply errors need resolution

# Outstanding Issues (CRITICAL)

## Issue 1: Escalation Delay Validation Error

Error: expected rule.0.escalation_delay_in_minutes to be at least (1), got 0

Affected Policies:

critical_infrastructure

environmental_systems

security_response

life_safety_critical

Solution: Change all escalation_delay_in_minutes = 0 to escalation_delay_in_minutes = 1 in escalation_policies.tf

## Issue 2: PagerDuty User License Limit

Error: License error: Unable to perform request at this time. account could not be locked

Cause: PagerDuty account has reached maximum user limit (attempted to create 15 users)

Solution Options:

Option A: Reduce number of demo users to fit within license limit

Option B: Use existing PagerDuty users instead of creating new ones

Option C: Upgrade PagerDuty account license

# System Architecture

## Alert Flow

Niagara BMS → Integration Layer → PagerDuty → CMMS (Work Orders)

## Alert Categories

Physical Security: Door breaches, access control, perimeter alerts

Environmental: Temperature, humidity, water detection

HVAC/Cooling: Cooling failures, airflow issues

Power/Electrical: UPS alerts, power distribution, generator status

Network Infrastructure: Switch failures, connectivity issues

# Teams & Escalation Structure

## Facilities Team

Responsibility: HVAC, environmental, physical infrastructure

Email: facilities-team@datacenter.com

## Security Team

Responsibility: Physical security, access control

Email: security-team@datacenter.com

## Network Operations

Responsibility: Network infrastructure, connectivity

Email: netops@datacenter.com

## Critical Response

Responsibility: Life safety, critical infrastructure

Email: critical-response@datacenter.com

# Escalation Policies

## Critical Infrastructure

Escalation Flow:
1. On-call engineer (immediate + 1 min)
2. Team lead (5 min)
3. Director (10 min)
4. VP Operations (15 min)

Repeat Loops: 9

## Environmental Systems

Escalation Flow:
1. Environmental specialist (immediate + 1 min)
2. Facilities manager (10 min)
3. Director (20 min)

Repeat Loops: 5

## Security Response

Escalation Flow:
1. Security officer (immediate + 1 min)
2. Security manager (5 min)
3. Security director (10 min)

Repeat Loops: 7

## Life Safety Critical

Escalation Flow:
1. Critical response team (immediate + 1 min, dual notification)
2. Emergency coordinator (3 min)
3. Site director (5 min)
4. Executive on-call (10 min)

Repeat Loops: 9

## HVAC Cooling

Escalation Flow:
1. HVAC technician (5 min)
2. Facilities lead (15 min)
3. Manager (30 min)

Repeat Loops: 3

## Network Operations

Escalation Flow:
1. Network engineer (5 min)
2. Senior engineer (15 min)
3. Network manager (30 min)

Repeat Loops: 3

# PagerDuty Services

## Physical Security Monitoring

Description: Door breaches, access violations

Alert Grouping: intelligent

Escalation Policy: security_response

## Environmental Monitoring

Description: Temperature, humidity, water detection

Alert Grouping: intelligent

Escalation Policy: environmental_systems

## HVAC & Cooling Systems

Description: Cooling failures, airflow issues

Alert Grouping: time

Escalation Policy: hvac_cooling

## Power & Electrical Systems

Description: UPS, PDU, generator alerts

Alert Grouping: intelligent

Escalation Policy: critical_infrastructure

## Network Infrastructure

Description: Network device failures

Alert Grouping: content_based

Escalation Policy: network_operations

# On-Call Schedules (Australia/Sydney)

Critical Infrastructure 24/7: Weekly rotation, 24/7 coverage (starts Monday 09:00)

Environmental Monitoring: Business hours (Mon-Fri 08:00-18:00) (starts Monday 08:00)

Security 24/7: Weekly rotation, 24/7 coverage (starts Monday 09:00)

HVAC On-Call: Business hours + extended (Mon-Sat 06:00-22:00) (starts Monday 06:00)

Network Operations 24/7: Weekly rotation, 24/7 coverage (starts Monday 09:00)

Facilities Management: Business hours (Mon-Fri 07:00-19:00) (starts Monday 07:00)

Executive Escalation: Weekly rotation, 24/7 coverage (starts Monday 09:00)

# Generated Files & Deliverables

## Interactive Demo

File: pagerduty-datacenter-demo.html

Features:

Visual alert flow diagram

Alert simulation with severity levels

Active incidents panel with countdown timers

Interactive incident management (acknowledge/resolve)

Alert history and statistics

## Terraform Infrastructure Code

Package: pagerduty-datacenter-demo-sydney-v2.zip

main.tf: Provider configuration and version constraints

variables.tf: Variable definitions (timezone, MS Teams channels)

terraform.tfvars: Variable values (requires PagerDuty API token)

users.tf: 15 demo users across all teams

teams.tf: 4 teams with member assignments

schedules.tf: 7 on-call schedules (Australia/Sydney timezone)

escalation_policies.tf: 6 escalation policies (NEEDS FIX)

services.tf: 5 services with alert grouping settings

outputs.tf: Output values for integration keys and IDs

.gitignore: Excludes sensitive files from version control

README.md: Setup and deployment instructions

# Next Steps (Action Required)

1. Fix escalation_policies.tf - change all escalation_delay_in_minutes = 0 to = 1

2. Reduce number of users in users.tf to fit PagerDuty license limit (or use existing users)

3. Add PagerDuty API token to terraform.tfvars

4. Run terraform plan to verify fixes

5. Run terraform apply to deploy infrastructure

6. Test alert flow using integration keys from outputs

7. Run demo scenarios using the HTML interactive demo

# Important Configuration Details

## Required: PagerDuty API Token

Location: terraform.tfvars

How to obtain: PagerDuty → User Icon → API Access → Create API Key (Full Access)

⚠️ Security: Never commit API token to version control (.gitignore is configured)

## Timezone Configuration

All schedules use: Australia/Sydney (UTC+11:00)

## MS Teams Integration

Channel: datacenter-incidents

Conference Bridge: MS Teams meeting URL format

# Demo Scenarios

## Critical Temperature Alert

Category: Environmental

Severity: critical

Escalation Flow: Immediate escalation to environmental specialist → facilities manager → director

## Door Breach

Category: Physical Security

Severity: high

Escalation Flow: Security officer → security manager → security director

## Cooling System Failure

Category: HVAC

Severity: critical

Escalation Flow: HVAC technician → facilities lead → manager

## UPS Battery Low

Category: Power/Electrical

Severity: warning

Escalation Flow: On-call engineer → team lead → director → VP

## Network Switch Down

Category: Network

Severity: high

Escalation Flow: Network engineer → senior engineer → network manager

# Technical Notes

## Terraform Provider Version

PagerDuty Provider: ~> 3.0

## Alert Grouping Types

intelligent: AI-based grouping (requires empty config block)

time: Time-based grouping (5-minute window)

content_based: Groups by alert content similarity

## Known Issues Fixed

✅ Deprecated alert_grouping_parameters replaced with pagerduty_alert_grouping_setting

✅ Intelligent grouping now includes empty config {} block

✅ Life safety critical num_loops reduced from 10 to 9 (max allowed)

⚠️ Escalation delay validation error (needs fix)

⚠️ User license limit error (needs resolution)

# Demo User Roster (15 Users)

| Name | Email | Team |
| --- | --- | --- |
| Alex Turner | alex.turner@datacenter.com | Facilities Team |
| Jordan Lee | jordan.lee@datacenter.com | Facilities Team |
| Sam Chen | sam.chen@datacenter.com | Facilities Team |
| Morgan Blake | morgan.blake@datacenter.com | Facilities Team |
| Casey Martinez | casey.martinez@datacenter.com | Security Team |
| Riley Johnson | riley.johnson@datacenter.com | Security Team |
| Taylor Swift | taylor.swift@datacenter.com | Security Team |
| Drew Anderson | drew.anderson@datacenter.com | Security Team |
| Chris Park | chris.park@datacenter.com | Network Operations |
| Jamie Wilson | jamie.wilson@datacenter.com | Network Operations |
| Avery Thompson | avery.thompson@datacenter.com | Network Operations |
| Quinn Davis | quinn.davis@datacenter.com | Network Operations |
| Reese Morgan | reese.morgan@datacenter.com | Critical Response |
| Skyler Brown | skyler.brown@datacenter.com | Critical Response |
| Dakota Kim | dakota.kim@datacenter.com | Critical Response |

Document Generated: 2026-03-09

Status: Ready for Terraform fixes and deployment